# SOP Template: Safety Inspection for Construction

Free safety inspection SOP for construction sites. Covers OSHA compliance, hazard identification, and daily site walkthroughs.

## Purpose

Conduct daily and weekly safety inspections on construction sites to identify hazards before they cause injuries, maintain OSHA compliance, and protect the company from citations that average $15,625 per serious violation. Inspections must be documented — verbal walkthroughs without written records don't count when OSHA asks for documentation.

## Scope

Covers daily pre-shift inspections, weekly comprehensive site walkthroughs, and post-incident inspections. Does not cover specialized inspections (crane, scaffold) which require competent person certification.

## Prerequisites

- OSHA 30-hour certification completed by the safety officer

- Site-specific safety plan on file with the general contractor

- PPE inventory stocked and accessible on site

- First aid kit and emergency action plan posted on site

- Inspection checklist template available in Procore or printed

## Roles & Responsibilities

Safety Officer

- Conduct weekly comprehensive site inspections

- Document findings in Procore safety module

- Issue stop-work orders for imminent danger conditions

Site Supervisor / Foreman

- Conduct daily pre-shift inspections before crews begin work

- Verify all workers have required PPE before entering the site

- Address hazards identified during the daily walkthrough immediately

Project Manager

- Review weekly safety inspection reports

- Approve spending for safety corrective actions

- Report OSHA-recordable incidents within required timeframes

## Procedure

### Step 1: Conduct the daily pre-shift inspection

Before any crew begins work each day, the site supervisor walks the active work areas and checks: fall protection in place for work above 6 feet, scaffolding properly erected and tagged, excavation shoring intact, electrical cords and GFCIs functional, housekeeping (clear walkways, materials stored properly), and weather conditions safe for work.

  - Walk all active work areas and access points

  - Verify fall protection for any work above 6 feet (guardrails, harnesses, nets)

  - Check scaffolding for proper base plates, cross bracing, and inspection tags

  - Test GFCIs on all temporary electrical circuits

  - Verify housekeeping — clear walkways, stacked materials stable

Warning: If you identify an imminent danger condition (unprotected edge, energized electrical exposure, unsupported trench), issue a stop-work order for that area immediately. Do not wait for the weekly inspection.

### Step 2: Verify PPE compliance

Before each shift, verify every worker on site has the required PPE: hard hat, safety glasses, high-visibility vest, steel-toed boots, and any task-specific PPE (gloves, hearing protection, fall harness). Workers without proper PPE do not enter the work area.

  - Check each worker's PPE at the site entry point

  - Verify fall harnesses are inspected and within service life (check tags)

  - Confirm hearing protection is available for work near loud equipment

  - Document any PPE violations and corrective actions taken

### Step 3: Conduct the weekly comprehensive inspection

Once per week, the safety officer conducts a thorough site inspection covering all OSHA focus areas: fall protection (29 CFR 1926.501), scaffolding (1926.451), excavations (1926.651), electrical (1926.405), struck-by hazards, and caught-in/between hazards. Document every finding with photos in Procore.

  - Fall protection: guardrails, personal fall arrest systems, hole covers

  - Scaffolding: competent person inspection tags, load limits, access ladders

  - Excavations: shoring, sloping, protective systems for trenches >5 feet

  - Electrical: GFCI protection, proper grounding, cord condition

  - Struck-by: hardhat compliance, barricades around overhead work, flagging for equipment

  - Caught-in/between: machine guarding, trench access, rotating equipment guards

Tip: Follow the OSHA top-10 most cited violations list — these are the hazards OSHA inspectors look for first. If you catch what they'd catch, you avoid the citation.

### Step 4: Document findings and assign corrective actions

For each hazard found, document in Procore's safety module: the hazard description, location on site, severity (imminent danger / serious / other), photo evidence, assigned person responsible for correction, and deadline. Imminent danger: correct immediately. Serious: correct within 24 hours. Other: correct within 1 week.

  - Enter each finding in Procore with photo and location

  - Classify severity per OSHA standards

  - Assign corrective action to the responsible foreman or subcontractor

  - Set deadlines: immediate / 24 hours / 1 week based on severity

### Step 5: Verify corrective actions and close findings

Before the next weekly inspection, the safety officer verifies all assigned corrective actions are completed. Re-inspect the specific location and photograph the corrected condition. Close the finding in Procore with verification photos and date. Findings not corrected by deadline are escalated to the project manager.

  - Re-inspect each finding location

  - Photograph the corrected condition

  - Close the finding in Procore with verification documentation

  - Escalate any overdue corrections to the project manager

### Step 6: Compile the weekly safety report

At the end of each week, compile the safety report: total findings, breakdown by category, open vs. closed items, any incidents or near-misses, and trend comparison to prior weeks. Deliver to the project manager and post a summary in the site trailer for crew visibility.

  - Summarize total findings by category and severity

  - Track open items and closure rate

  - Note any incidents, near-misses, or OSHA-recordable events

  - Compare trends to previous weeks

  - Post summary in the job site trailer

## Completion Checklist

☐  Daily pre-shift inspection completed before work begins

☐  PPE compliance verified for all workers

☐  Fall protection in place for all work above 6 feet

☐  Scaffolding inspection tags current

☐  Excavation protective systems in place for trenches >5 feet

☐  GFCIs tested and functional on all temporary circuits

☐  Weekly comprehensive inspection completed and documented in Procore

☐  All findings photographed with severity classification

☐  Corrective actions assigned with deadlines

☐  Overdue corrections escalated to project manager

☐  Weekly safety report compiled and distributed

## Key Performance Indicators

| Metric | Target |
| --- | --- |
| Daily inspection completion | 100% of work days |
| Imminent danger correction time | Immediate (before work continues) |
| Serious finding correction time | Within 24 hours |
| OSHA-recordable incident rate | Below industry average (2.8 per 100 workers) |

## Revision Schedule

Quarterly, or immediately after any OSHA inspection, serious incident, or change in site conditions.

Free SOP template by Glyde · glydehq.com · Record it once, keep it current.